DVT Firmware Pack — Usage Guide

Board: nRF5340 BLE dev board Rev B (MDBT53-1M + RP2040 probe), fab 2026-07-04. Pack contents: the DVT_Helper Zephyr app, the stock RP2040 debugprobe UF2 (external download) and this guide. Pin maps use the verified fab-data pins (Appendix B of the DVT plan) — nothing to edit.

| Firmware | Covers | When |
| --- | --- | --- |
| debugprobe_on_pico.uf2 (stock, >= 2.2) | DBG-01..10 — CMSIS-DAP probe + CDC UART bridge on the RP2040 | Right after PWR passes |
| DVT_Helper (app core) | NRF-01..09, GPIO-01..05, I2C-01..05, IMEAS-01..09, SYS-01..03 — the workhorse | After DBG-05 unlock |
| ipc_radio (net core, built by sysbuild) | RF-01..07 + IMEAS-07 — BLE controller | When you reach RF |

# 1. One-Time Setup

- nRF Connect SDK v2.9+ (v3.x fine) with the toolchain — the standard NCS install. Smoke-build the app once before DVT day.

- probe-rs (recommended flasher for this board): irm/curl installer from probe.rs — handles the nRF5340 app core, net core, and the APPROTECT recover, all over CMSIS-DAP.

- Optional: pyOCD >= 0.38 (west flash --runner pyocd works for the app core) and OpenOCD. Debugging: VS Code Cortex-Debug extension with pyocd or probe-rs gdb-server.

- RP2040 side: download debugprobe_on_pico.uf2 (release >= 2.2) from the raspberrypi/debugprobe GitHub releases. It must be the on_pico build — the on_debugprobe build uses different pins.

THE THING TO KNOW FIRST: the nRF Connect VS Code extension Flash/Debug buttons call nrfutil/J-Link and will never see this board’s CMSIS-DAP probe. Everything flashes from the terminal. This is a property of Nordic’s tooling, not a board fault — it goes on page one of the product docs too.

# 2. RP2040: Flashing the Probe (DBG-01..02)

- SW1 to DEBUG. Plug USB-C. A blank board boots straight to the ROM bootloader: an RPI-RP2 drive appears. (Later, to get back: hold SW2 = BOOTSEL, tap SW3 = RUN, release SW2.)

- Drag debugprobe_on_pico.uf2 onto RPI-RP2. The board re-enumerates as "Debugprobe on Pico (CMSIS-DAP)" — one CMSIS-DAP interface + one CDC serial port (VID:PID 2E8A:000C). LED3 = probe activity.

- The CDC port is the nRF console for the whole campaign (115200 8N1). Pin mapping is stock: GPIO2 SWCLK, GPIO3 SWDIO, GPIO4/5 UART — matching this board’s netlist exactly.

# 3. nRF5340: Unlock, Build, Flash (DBG-05..06, NRF-01..02)

Build (from the DVT_Helper folder):

west build -b nrf5340dk/nrf5340/cpuapp --sysbuild

#  outputs: build/merged.hex (app core)

#           build/merged_CPUNET.hex (net core BLE controller)

The board is DK-pin-compatible (uart0 / i2c1 / MX25R64 QSPI). The one deviation — LFXO internal 6 pF for the cap-less NX3215SA — is already set in prj.conf. Do not "fix" it back to the DK default.

First contact with a virgin module — APPROTECT unlock (DBG-05):

Fresh nRF5340s ship access-port-protected. The first connect fails or shows only CTRL-AP. Run the recover once per board:

probe-rs erase --chip nRF5340_xxAA --allow-erase-all

This issues the CTRL-AP ERASEALL (chip + UICR wiped) and debug opens. NCS firmware keeps it open on subsequent boots for development builds.

Flash:

# app core (wired tests — enough for everything except RF):

probe-rs download --chip nRF5340_xxAA --binary-format hex build/merged.hex

probe-rs reset --chip nRF5340_xxAA

#   (alternative for the app core: west flash --runner pyocd)

# net core, when you reach the RF section:

probe-rs download --chip nRF5340_xxAA --binary-format hex build/merged_CPUNET.hex

probe-rs reset --chip nRF5340_xxAA

Net-core flashing over CMSIS-DAP is the roughest edge of this toolchain: probe-rs is the path that works; pyOCD/OpenOCD app-core-only is fine for the wired sections. Fallback if a unit misbehaves: a J-Link on the J4 TC2030 pads flashes both cores with stock Nordic tools (west flash). Budget a little time here on board #1 and record the exact tool versions on the tracker Dashboard.

Open the probe CDC port at 115200 (or an RTT viewer — both shells run simultaneously). You should see the banner; type dvt for the command tree. dvt info prints the FICR Device ID — that is the board S/N for the tracker.

# 4. The Helper Shell, Test by Test

| Command | DVT IDs | How to run it |
| --- | --- | --- |
| dvt info | NRF-02/03 | S/N (FICR), reset reason, LFCLK source. Record S/N on the Dashboard. |
| dvt clk cal | NRF-04/05 | 8 s automated LFXO-vs-HFXO ppm measurement. Big error = wrong load-cap setting (must be internal 6 pF). |
| dvt gpio walk | GPIO-01 | Each header GPIO drives high ~1.5 s in turn; DMM the announced pin, spot-check neighbours. "walk all" adds the UART/I2C pins — console dies at H1.15, watch on RTT, reset (SW4) after. |
| dvt gpio in | GPIO-02 | Pull-up input table, 20 samples. Jumper each pin to GND and watch its column drop. |
| dvt gpio pattern | GPIO-03 | Fully automatic adjacent-bridge test; PASS/FAIL per pair, no instruments. |
| dvt adc | GPIO-04 | AIN0..7 in mV, 10 rounds. Feed 2.000 V through ~10 k to each AIN and compare to the DMM. |
| dvt i2c scan / sht4x [fast] | I2C-02..04 | Qwiic scan then SHT4x read with CRC; add "fast" for 400 kHz. R40/R41 must be fitted (BOM gap). |
| dvt qspi id / test / dpd | NRF-06/07, IMEAS-06 | Driver-verified JEDEC C2 28 17; erase-write-verify of the last sector; DPD toggle while watching the J2 meter. |
| dvt usb | NRF-08 | Slide SW1 to APP first; nRF native CDC enumerates. Back to DEBUG after (console lives on the probe). |
| dvt ble adv [0¦3] / stop | RF-01..05 | Advertise as nRF5340-DVT at 0 or +3 dBm. Fails with a clear message if the net core is blank. |
| dvt cpu | IMEAS-03 | 3 s warning, then a 30 s busy-loop — read the J2 meter. |
| dvt sleep idle / off | IMEAS-04/05 | 5 s countdown, console + QSPI suspend, then System ON idle or System OFF. RTT survives idle; recover with SW4. Detach the debugger first or the floor reads high (IMEAS-08). |
| dvt demo | SYS-01/03 | SHT4x poll + BLE adv soak loop until reset. Log the start time. |

# 5. Low-Power Measurement Notes (IMEAS)

- J2: jumper out, meter in series. The pin nearer U7 is the LDO side (marked in PRE-01). PPK2 in ampere-meter mode is the reference instrument; a DMM works but mind burden voltage on uA ranges (IMEAS-01 checks it).

- The measured domain is exactly: MDBT53 module + MX25R64 flash + QSPI/I2C pull-ups + J4 Vref pin. The LDO’s own quiescent current is upstream of J2 and is NOT in your number — the reading is pure load.

- Detach pyocd/probe-rs/gdb before any floor measurement; an attached session holds the debug power domain up. IMEAS-08 documents the delta on purpose.

- Idle-high I2C/QSPI pull-ups draw zero; a Qwiic device holding a line low would show up — unplug peripherals for the floor numbers.

- Record everything: these first-article numbers become the datasheet sleep-current claims.