Design Verification Test (DVT) Plan

nRF5340 BLE Development Board (MDBT53-1M + RP2040 CMSIS-DAP probe) — Rev B

| Field | Value |
| --- | --- |
| Board | nRF5340 BLE dev board, 66 x 25 mm, 6-layer. Raytac MDBT53-1M (nRF5340) + RP2040 debug probe, LiPo charge/protect, isolated 3V3_NRF rail with J2 current-measurement header. |
| Fab data | Netlist_Schematic1_2026-07-04.tel, BOM_Board1_PCB1_20260704.xlsx, PickAndPlace_PCB1_2026_07_04.xlsx (66 x 25 mm outline). |
| Assembly | JLCPCB bare boards; first articles (3-5 pcs) hand hot-plate reflow, USB-C J1 hand-soldered after reflow. H1/H2/J2 headers + R40/R41 are kitting items (see section 1). |
| Doc version | 2.0 — 2026-07-22. Regenerated against the 2026-07-04 fab set with the DVT_Helper firmware pack; supersedes v1.x (2026-07-05). Companion firmware: DVT_Helper 1.0 (nRF5340) + stock debugprobe_on_pico.uf2 >= 2.2 (RP2040). |
| Board S/N under test | ____________ (use the nRF FICR Device ID — see NRF-02) |
| Tester / Date | ____________ / ____________ |
| Test firmware version | ____________ |
| Companion results file | nRF5340_RevB_DVT_Tracker.xlsx — per-measurement results, pass/fail rollup and issue log. Plan Ref column there maps 1:1 to the test IDs below. |

Table of Contents

# 0. How to Use This Document

Run sections in order. VIS and PRE are unpowered gates: any failure there means do not apply power until it is resolved. First power-up (PWR-01) uses a current-limited bench supply into the +5V header pin — not a USB port on your computer.

Each test has an ID, procedure, expected result and blank Measured / P-F fields. Record actual numbers, not just pass/fail — measured values are what make first article #1 comparable to unit #40 of the pilot run. Execution results live in the companion tracker; section 17 carries the rollup.

Stop-on-fail rules:

- Any rail-to-GND short in PRE — stop.

- Current limit trips at first power-up — stop and thermal-inspect.

- Never connect a LiPo until the battery-pin no-short check (PRE-11) and the reverse-polarity test (BAT-01, bench supply) have passed.

- IMEAS numbers are only valid with the debug session detached and the J2 jumper replaced by the meter — see section 14 preamble.

# 1. References and Revision Notes

Source data: the 2026-07-04 netlist (98 nets / 98 packages), BOM (42 lines) and pick-and-place (98 placements) were re-reconciled line-by-line for this revision. Findings, in severity order:

X  R40 / R41 missing from the BOM (blocking for I2C).

The 4.7 k I2C pull-ups (bottom side, x=22.1/20.8, y=4.3) are in the netlist and pick-and-place but absent from the BOM. Kitted from the BOM alone, the Qwiic bus has no pull-ups and every I2C test fails. Why it matters: this is the same failure class as the C53/C54 omission caught last revision — the BOM export dropped parts the layout carries. Fix: hand-fit 2x 4.7 kOhm 0402 (e.g. LCSC RC0402FR-074K7L class) before the I2C section, and add the line to the BOM before the pilot run.

OK  CN1 Qwiic power rail regression is FIXED.

CN1.2 is on 3V3_OTHER in the 2026-07-04 netlist (verified member-by-member). Qwiic devices no longer load the measured rail; I2C-05 and IMEAS-09 prove it on hardware.

OK  C53 / C54 (nRF DECD / DECR) present in BOM line 1; module confirmed MDBT53-1M (BOM line 40, netlist package).

Remaining known items carried into DVT:

- H1 / H2 (19-pin headers) and J2 are in netlist + pick-and-place but not the BOM — deliberate kitting items. Procure and hand-solder before VIS-07.

- J4 is the TC2030-NL footprint: DNI by design, never a BOM line. Cable = TC2030-CTX-NL.

- C49 / C50 (NFC tuning, bottom) are DNI; the schematic placeholder value "100 nF" must not be populated (VIS-09). NFC tuning belongs to the customer antenna.

- Test-pad naming: netlist calls the three PTH pads NFC1 / NFC2 / GND; pick-and-place calls them TP1 / TP2 / TP3. Geometric mapping TP1=NFC1, TP2=NFC2, TP3=GND — PRE-13 confirms by continuity.

- P1.04 (U14.11) is a one-pin stub net — schematic housekeeping, harmless to this build.

- FCC / ISED "Contains" module marking is absent from the silkscreen (VIS-10 records). Not a DVT blocker; it is a sale blocker — fix before the pilot run.

- Raytac antenna keep-out sign-off (service@raytac.com) still pending at plan date; RF-02/03 give the empirical check meanwhile.

- Virgin MDBT53 modules ship APPROTECT-locked: the first SWD connect needs a CTRL-AP ERASEALL (DBG-05). This is normal, not a fault.

# 2. Required Equipment

| # | Item | Used for |
| --- | --- | --- |
| 1 | DMM with diode mode | Everything |
| 2 | Bench PSU, 0-5 V, adjustable current limit (CC mode) | First power, battery emulation, brownout sweep, reverse-polarity test |
| 3 | USB-C cables (A-to-C and C-to-C) + inline USB power meter | USB validation, input / charge current |
| 4 | 1S LiPo with JST-XH plug (~500 mAh) — verify pigtail polarity first | Battery tests (only after BAT-01) |
| 5 | uA-capable current meter — Nordic PPK2 (ampere-meter mode) ideal; uCurrent or DMM uA range acceptable | J2 measurements, IMEAS section |
| 6 | 2x 4.7 kOhm 0402 resistors | Hand-kit R40/R41 (BOM gap — see section 1) |
| 7 | Qwiic SHT4x breakout + cable | I2C port test (same unit as the ESP32 project) |
| 8 | Phone with nRF Connect (Nordic) | RF section |
| 9 | Reference BLE board — nRF5340-DK ideal; any known-good BLE advertiser as fallback | Comparative RSSI (RF-03) |
| 10 | PC with NCS >= 2.9 toolchain, probe-rs (and/or pyOCD >= 0.38), a UF2 of debugprobe_on_pico >= 2.2 | DBG + NRF sections. The nRF Connect VS Code Flash/Debug buttons do NOT work with this probe — terminal only. |
| 11 | USB-UART adapter (3.3 V) | NRF-08 header UART observation |
| 12 | Oscilloscope (optional) | Ripple, switchover dip |
| 13 | IR thermometer / thermal camera (optional) | Charger and LDO thermal |
| 14 | TC2030-CTX-NL cable + external probe (optional) | DBG-10 external SWD path |

# 3. Safety Notes

- LiPo: JST-XH pigtail polarity is not standardized between vendors. Verify with the DMM that battery + lands on J5.1 (net through Q3 to BATT+) before ever plugging in. Q3 gives reverse protection — tested in BAT-01 — but do not rely on it.

- Use the bench PSU in current-limit mode for every "first": first power, first battery connect, first USB.

- First power goes in through H1.1 (+5V pin): it passes F1 (0.5 A polyfuse) and D2, the gentlest path on the board.

- ESD precautions: the MDBT53 antenna end and the 0.4 mm RP2040 are the sensitive spots. High-humidity shop: assemble MSL parts within ~1 week of opening bags or bake first.

- USB-C J1 is hand-soldered after reflow (hybrid SMD/TH shell legs) — inspect before first USB plug-in (VIS-06).

# 4. Board Reference Data

## 4.1 Power Tree (as built)

USB-C J1 VBUS -> V_USB -(D3 B5819WS)-> V_SYS -+-> U6 AP2112K-3.3 -> 3V3_OTHER

|                             +-> U7 AP2112K-3.3 -> J2.1 ][ J2.2 -> 3V3_NRF

|                                  (J2 = current-measurement header, jumper = normal)

+-> U5 MCP73831 (PROG R5 5.1k ~196 mA) -> BATT+ -(Q3 AO3415 rev-pol)-> J5.1

+-> Q1 gate (AO3407): USB present -> Q1 OFF; USB absent (R7 10k) -> BATT+ -> V_SYS

+5V pin H1.1 -(F1 0.5A polyfuse)-(D2 B5819WS)-> V_SYS   (input only; D2 blocks output)

Battery low side: J5.2 (BATT-) -(Q2 FS8205A dual FET)- GND, controlled by U8 DW01A

Rails: 3V3_OTHER = RP2040 + U3 flash + FSUSB42 + USBLC6 + CN1 Qwiic + H2.1 pin

3V3_NRF   = MDBT53 module + U4 MX25R64 QSPI + R31/32/33 QSPI PUs + R40/41 I2C PUs + J4.1 Vref

LEDs:  LED2 green = V_SYS power (R34 2.2k). LED1 yellow = charge STAT (R6 470).

LED3 yellow = RP2040 GPIO25 probe activity (R39 470). No LED on the measured rail.

## 4.2 Expected Voltages Summary

| Net | Condition | Expected | Where to probe |
| --- | --- | --- | --- |
| V_USB | USB-C 5 V source | 4.75-5.25 V | C2 + pad (50.9, 9.3) |
| V_SYS | USB powered | V_USB - 0.15...0.45 V | D2.1 (47.5, 6.3) / D3.1 |
| V_SYS | Battery only | V_BAT - <60 mV (Q1 on) | same |
| 3V3_OTHER | any power | 3.20-3.40 V | H2.1 header pin |
| 3V3_NRF | any power, J2 jumpered | 3.20-3.40 V | J2.2 (rail side) |
| BATT+ | battery connected | 3.0-4.23 V | C12 + pad (50.1, 21.9) |
| BATT+ | charging, near full | 4.15-4.25 V (VREG) | C12 |
| +5V (H1.1) | USB powered, pin unused | ~0 V (input-only) | H1.1 |
| SDA / SCL | idle | ~3.3 V (R40/R41 4.7k to 3V3_NRF) | CN1.3 / CN1.4 |
| REST | idle | ~3.3 V (module pull-up) | H1.19 |
| Charger PROG | while charging | ~1.0 V | U5 pin 5 / R5 |

## 4.3 Module Pin Map (MDBT53-1M / U14, as built)

GPIO assignments verified against the Zephyr nrf5340dk devicetree in the Rev B design review; uart0, i2c1 and the QSPI flash match the nRF5340-DK exactly, so stock DK builds run with only the LFXO overlay.

| Mod pin | GPIO | Net | Destination |
| --- | --- | --- | --- |
| 2 / 4 / 6 | SWDCLK / SWDIO / nRESET | SWCLK / SWDIO / REST | RP2040 GPIO2/3 via R29/R30 27R; J4 TC2030; SW4; H1.19 |
| 13 / 25 | P0.20 / P0.22 | NRF_TXD / NRF_RXD | uart0 <-> RP2040 GPIO5/4 via R36/R35 1k; H1.15 / H1.14 |
| 31 / 33 | P1.03 / P1.02 | SCL / SDA | i2c1 -> CN1 Qwiic + H1.17/18; R41/R40 4.7k to 3V3_NRF |
| 16 / 26 / 21 / 27 / 23 / 28 | P0.18 / P0.17 / P0.13 / P0.14 / P0.15 / P0.16 | NRF_QSPI CS/CLK/IO0-3 | U4 MX25R6435F; R31 (CS), R33 (IO2/WP), R32 (IO3/HOLD) 10k pull-ups |
| 46 / 48 / 44 | D+ / D- / VBUS | NRF_USB_P/N, R28 net | FSUSB42 APP channel; VBUS via R28 2.2R + C48 4.7u |
| 34 / 36 | XL2 / XL1 | X2 crystal | NX3215SA 32.768 kHz, CL 6 pF, NO external caps -> internal 6 pF in firmware |
| 32 / 30 | P0.02 / P0.03 (NFC) | NFC1 / NFC2 | TP1 / TP2 pads; C49/C50 DNI |
| 58 / 52 | DECD / DECR | - | C53 / C54 100 nF (BOM line 1) |
| 38 / 41 | VDD | 3V3_NRF | the measured rail |
| 11 | P1.04 | stub | no connection (housekeeping) |
| 37/39/35/20/9/7/5/60/65/57/56/45/43/59/55/53/49/47/17/15/29/19/18 | P0.04..P1.15 | header GPIO | H1 / H2 — see 4.4 |

## 4.4 Connector Pinouts (as built)

H1 (bottom edge, y=1.3, pin 1 = +5V end): 1 +5V(in) - 2 GND - 3 P0.29 - 4 P0.07/AIN3 - 5 P0.25/AIN4 - 6 P0.26/AIN5 - 7 P0.27/AIN6 - 8 GND - 9 P0.10 - 10 P0.09 - 11 P0.08 - 12 GND - 13 P1.01 - 14 P0.22(UART RX) - 15 P0.20(UART TX) - 16 GND - 17 P1.03(SCL) - 18 P1.02(SDA) - 19 REST

H2 (top edge, y=24.1, pin 1 = 3V3 end): 1 3V3(3V3_OTHER) - 2 GND - 3 P0.11 - 4 P0.12 - 5 P0.05/AIN1 - 6 P0.06/AIN2 - 7 P0.04/AIN0 - 8 GND - 9 P1.00 - 10 P1.15 - 11 P1.14 - 12 P1.12 - 13 GND - 14 P0.31 - 15 P0.28/AIN7 - 16 P1.11 - 17 P0.30 - 18 P1.10 - 19 GND

J2 current-measurement header (45.1, 21.5): pin nearer U7 = LDO output (J2.1), other = 3V3_NRF rail (J2.2). Jumper fitted = normal operation; jumper out + meter in series = measurement (PRE-01 marks pin 1).

J4 Tag-Connect TC2030-NL (20.9, 11.5): 1 3V3_NRF(Vref) - 2 SWDIO - 3 REST - 4 SWCLK - 5 GND - 6 NC. DNI footprint; cable TC2030-CTX-NL.

J5 battery JST-XH (57.9, 21.6): 1 BAT+ (via Q3) - 2 BAT- (via Q2/DW01A to GND). CN1 Qwiic (22.4, 5.2): 1 GND - 2 3V3_OTHER - 3 SDA - 4 SCL (standard Qwiic order: black, red, blue, yellow).

TP pads (x=15.9): TP1 NFC1 (y=10.4) - TP2 NFC2 (y=7.9) - TP3 GND (y=5.4). SW1 slide = USB routing: DEBUG -> RP2040, APP -> nRF native USB. SW2 RP2040 BOOTSEL - SW3 RP2040 RUN - SW4 nRF RESET.

# 5. VIS — Incoming Visual & Mechanical Inspection

Per board, under magnification. Hand hot-plate build: you are your own AOI — slow down on the fine-pitch parts.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| VIS-01 | Overall reflow quality under magnification: tombstones, missing 0402s, bridges (hot-plate build — check paste-starved corners first) | None found |  |  |
| VIS-02 | D2, D3 (B5819WS) polarity: cathode line toward V_SYS on both (D2 cathode = pad toward U6; D3 cathode = pad toward board interior) | Both correct |  |  |
| VIS-03 | Pin-1 / orientation vs silk: U1, U2, U3, U4, U5, U6, U7, U8, Q1, Q2, Q3 | All correct |  |  |
| VIS-04 | U14 MDBT53-1M: aligned, castellations wetted, antenna end (left board edge) clean — no solder, flux, labels or debris in the keepout | Clean |  |  |
| VIS-05 | Fine-pitch: U2 LQFN-56 0.4 mm, U4 WSON-8, U1 MSOP-10 — no bridges, fillets visible at pad edges | Clean |  |  |
| VIS-06 | USB-C J1 (hand-soldered after reflow): 12 signal pins wetted, no bridges, shell legs filled | Clean |  |  |
| VIS-07 | Through-hole H1/H2/J2/J5/SW1 seated flat, joints filled (H1/H2/J2 are kitted separately — not on the BOM) | Seated, filled |  |  |
| VIS-08 | SW2/SW3/SW4 click mechanically; SW1 slides with a detent in both positions | All OK |  |  |
| VIS-09 | C49/C50 (NFC tuning, bottom) NOT populated — DNI by design; the schematic '100 nF' value is a placeholder | Both empty |  |  |
| VIS-10 | Silkscreen legible; FCC/ISED 'Contains' module marking present? (known Rev B gap — record the result) | Record |  |  |
| VIS-11 | Breadboard fit: H1/H2 rows seat across a standard breadboard on 2.54 mm centers | Seats cleanly |  |  |

# 6. PRE — Unpowered Electrical Checks (DMM)

No power, no battery, J2 jumper fitted unless a step says otherwise. In resistance mode capacitor charging makes readings climb — note the settled trend, not the first flash.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| PRE-01 | Identify & mark pin 1s by continuity: J2 pin nearer U7.5 = LDO side (J2.1); H1.1 = +5V end (to F1); H2.1 = 3V3 end (to U6.5) | Mapped + marked |  |  |
| PRE-02 | V_USB to GND resistance (probe C2 + pad) | ~10 kOhm (R7), not < 1 k |  |  |
| PRE-03 | V_SYS to GND (probe D2.1 cathode pad) | High / climbing (>= 50000 Ohm) |  |  |
| PRE-04 | 3V3_OTHER to GND (probe H2.1) | > 5 kOhm and climbing |  |  |
| PRE-05 | 3V3_NRF to GND (probe J2.2 side) | > 5 kOhm and climbing |  |  |
| PRE-06 | J2.1 to J2.2 with jumper REMOVED — the measured rail must only connect through J2 | Open / > 1 MOhm |  |  |
| PRE-07 | Diode mode V_USB to V_SYS (red D3.2, black D3.1): B5819WS forward | 0.15-0.45 V |  |  |
| PRE-08 | Diode mode +5V input path (red H1.1, black D2.1): F1 + D2 forward | 0.15-0.50 V |  |  |
| PRE-09 | Reverse diode checks: red D2.1, black H1.1 = OL; red D3.1, black D3.2 = OL | OL both |  |  |
| PRE-10 | USB D+ to D- (D1 pads) and VBUS to D+/D-: no shorts | OL / > 1 MOhm |  |  |
| PRE-11 | Battery pins J5.1 to J5.2: no short; J5.2 (BATT-) to GND: not 0 Ohm (protection FETs unbiased) | No short |  |  |
| PRE-12 | SWDIO (J4.2) and SWCLK (J4.4) to GND: no short. REST (H1.19) to GND: no short; ~0 Ohm only while SW4 held | OK |  |  |
| PRE-13 | TP3 to GND continuity ~0 Ohm (confirms map TP1=NFC1, TP2=NFC2, TP3=GND); TP1/TP2 NOT shorted to GND | Confirmed |  |  |

# 7. PWR — First Power-Up and Power Rails

First power goes through the +5V header pin (F1 + D2 path) from a current-limited bench supply. USB only after PWR-06 passes.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| PWR-01 | First power: bench PSU 5.00 V, 100 mA CC limit, + to H1.1, - to H1.2 (GND). Supply current after 5 s | Stable, no CC trip (2-60 mA) |  |  |
| PWR-02 | Nothing hot after 60 s at first power (finger/IR: U6, U7, U2, U14) | Cool / barely warm |  |  |
| PWR-03 | V_SYS (D2.1) = PSU minus F1 + D2 drop | 4.3-4.9 V |  |  |
| PWR-04 | 3V3_OTHER at H2.1 | 3.20-3.40 V |  |  |
| PWR-05 | 3V3_NRF at J2.2 (jumper fitted) | 3.20-3.40 V |  |  |
| PWR-06 | J2 jumper contact drop: V(J2.1) - V(J2.2) | <= 5 mV |  |  |
| PWR-07 | Remove PSU. First USB power through inline USB meter, SW1 = DEBUG: V_USB | 4.75-5.25 V |  |  |
| PWR-08 | USB idle input current (blank chips: RP2040 in BOOTSEL ROM, nRF blank) | Record (5-100 mA) |  |  |
| PWR-09 | RPI-RP2 mass-storage device enumerates (blank U3 -> ROM bootloader; also proves X1 12 MHz and the FSUSB42 DEBUG path) | Enumerates |  |  |
| PWR-10 | SW1 to APP: RPI-RP2 disappears (bus routed to blank nRF, no device). Back to DEBUG: reappears | Behaves |  |  |
| PWR-11 | LED2 (green) on whenever V_SYS is present, off when unpowered | Correct |  |  |
| PWR-12 | Backdrive: USB powered, voltage at H1.1 (+5V pin) — D2 blocks; the pin is input-only | ~0 V |  |  |
| PWR-13 | Ripple (scope, optional): 3V3_OTHER and 3V3_NRF at idle | < 30 mVpp, record |  |  |
| PWR-14 | Brownout: PSU into H1.1, ramp 5.0 -> 3.6 -> 5.0 V slowly: rails sag and recover cleanly, no latch-up, RP2040 re-enumerates | Clean recovery |  |  |

# 8. CHG — Battery Charger (MCP73831-2AC, PROG = 5.1 k)

USB powered. Use the bench supply as a battery simulator first; a real cell only after BAT-01 has passed on this board.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| CHG-01 | LED1 (yellow) on while charging, off / hi-Z at termination | Correct |  |  |
| CHG-02 | Charge current into a cell at ~3.7 V (PROG R5 = 5.1 k -> ~196 mA). Series meter in battery lead, or USB-meter delta | 150-215 mA |  |  |
| CHG-03 | Termination voltage: BATT+ (C12 + pad) as charge completes | 4.15-4.25 V |  |  |
| CHG-04 | U5 temperature during full-rate charge | Warm, not untouchable (<= 70 degC) |  |  |
| CHG-05 | No-battery behavior: LED1 may flicker / cycle (MCP73831 no-load hiccup) — normal, document it | Documented |  |  |
| CHG-06 | Recharge: run the board on battery briefly, replug USB: charge cycle restarts (LED1 on) | Restarts |  |  |

# 9. BAT — Battery Protection and Power Path (DW01A + FS8205A + AO3415/AO3407)

BAT-01 (reverse test, bench supply) comes before the first real cell — no exceptions.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| BAT-01 | REVERSE battery test with bench PSU reversed at J5, 50 mA CC — BEFORE ever fitting a real cell. Current flow | Q3 blocks; board dead, no heat |  |  |
| BAT-02 | Correct polarity, battery only (USB out): board runs. Drop V_BAT - V_SYS (Q1 enhanced) | <= 60 mV |  |  |
| BAT-03 | Q3 drop under load: V(C12 BATT+) - V(J5.1) while running on battery | <= 30 mV |  |  |
| BAT-04 | Switchover battery -> USB: plug USB while running; RP2040 CDC does not drop, no reset (scope V_SYS dip optional — record) | No reset |  |  |
| BAT-05 | Switchover USB -> battery: unplug USB; keeps running | No reset |  |  |
| BAT-06 | DW01A under-voltage cutoff: PSU as battery, step down 0.1 V: load disconnects at; recovers when raised above ~3.0 V | 2.30-2.55 V |  |  |

# 10. DBG — RP2040 Debug Subsystem (CMSIS-DAP probe + UART bridge)

The RP2040 runs stock raspberrypi/debugprobe firmware (debugprobe_on_pico.uf2, release >= 2.2) — no custom probe firmware. SW1 stays in DEBUG for this section.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| DBG-01 | Flash probe firmware: SW1=DEBUG; RPI-RP2 drive present (blank flash) or hold SW2 + tap SW3; copy debugprobe_on_pico.uf2 (release >= 2.2) | UF2 accepted, reboots |  |  |
| DBG-02 | Re-enumeration: CMSIS-DAP (VID:PID 2E8A:000C, 'Debugprobe on Pico') plus a CDC serial port | Both interfaces |  |  |
| DBG-03 | LED3 (yellow) behaves per debugprobe firmware (activity at connect / traffic) | OK |  |  |
| DBG-04 | probe-rs list (or pyocd list) shows the probe | Listed |  |  |
| DBG-05 | First SWD connect: virgin modules ship APPROTECT-locked. Run probe-rs erase --chip nRF5340_xxAA --allow-erase-all once (CTRL-AP ERASEALL); then connect reads core IDs | Unlock + connect OK |  |  |
| DBG-06 | Flash DVT_Helper app core (probe-rs download merged.hex, or west flash --runner pyocd): banner prints | Flashed, banner |  |  |
| DBG-07 | UART bridge: shell responds on the probe CDC at 115200 (proves R35/R36 crossover on RP2040 GPIO4/5) | Shell works |  |  |
| DBG-08 | SW4 resets the nRF only (banner reprints, CDC stays). SW3 resets the RP2040 only (CDC re-enumerates, nRF untouched) | Independent resets |  |  |
| DBG-09 | SW2 + SW3 returns the RP2040 to BOOTSEL (field UF2 update path); reflash UF2 | RPI-RP2 back |  |  |
| DBG-10 | Tag-Connect J4 continuity: J4.1=3V3_NRF, J4.2=U14.4 SWDIO, J4.3=REST, J4.4=U14.2 SWCLK, J4.5=GND. If a TC2030-CTX-NL + external probe is on hand: external session works with the RP2040 idle | Verified |  |  |

# 11. NRF — nRF5340 Bring-Up (MDBT53-1M)

Firmware = DVT_Helper (see Appendix C and the Firmware Guide). Build once before DVT day. Console: the probe CDC port at 115200, or RTT.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| NRF-01 | Build DVT_Helper: NCS >= 2.9, west build -b nrf5340dk/nrf5340/cpuapp --sysbuild. Only deviation from stock DK: LFXO internal 6 pF (in prj.conf) | Builds clean |  |  |
| NRF-02 | dvt info: FICR Device ID prints — record as board S/N on the Dashboard; reset reason sane | S/N recorded |  |  |
| NRF-03 | dvt clk: LFCLK source = XTAL (X2 running on internal 6 pF caps) | XTAL |  |  |
| NRF-04 | dvt clk cal: LFXO frequency error vs HFXO | ¦ppm¦ <= 150 (target +/-50) |  |  |
| NRF-05 | dvt qspi id: MX25R6435F ready — driver verifies JEDEC C2 28 17 at init; flash is on 3V3_NRF | Ready / PASS |  |  |
| NRF-06 | dvt qspi test: erase / write / verify last 4 K sector | PASS |  |  |
| NRF-07 | dvt usb with SW1=APP: nRF native CDC enumerates (FSUSB42 APP path + R28/C48 VBUS sense). SW1 back to DEBUG after | Enumerates |  |  |
| NRF-08 | Console also visible at H1.15 (NRF_TXD) with a 3.3 V USB-UART adapter, RX only — do not drive H1.14 against the probe | Output seen |  |  |
| NRF-09 | Clean boot: debug session closed, SW4 tap: banner in < 1 s | Clean boot |  |  |

# 12. GPIO — I/O Verification (H1 / H2 headers)

DVT_Helper gpio commands drive the tests; only instruments are a DMM and a jumper wire.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| GPIO-01 | dvt gpio walk: each header GPIO drives high in turn; DMM the announced pin, spot-check neighbours (~0 V). Use 'walk all' + RTT to include the UART/I2C pins, reset after | 3.2-3.4 V driven |  |  |
| GPIO-02 | dvt gpio in: all 1s at rest (pull-ups); each pin reads 0 when jumpered to GND | Correct |  |  |
| GPIO-03 | dvt gpio pattern: adjacent-bridge auto test on all physically adjacent signal pairs | PASS all pairs |  |  |
| GPIO-04 | dvt adc with 2.000 V fed through ~10 k to each AINx in turn (PSU + DMM). Reading at 2.000 V | 1940-2060 mV |  |  |
| GPIO-05 | Header power pins: +5V at H1.1 (~0 V on USB, input-only), 3V3_OTHER at H2.1, every GND pin 0 Ohm to GND | Correct |  |  |

# 13. I2C — Qwiic Port (CN1) and I2C1

Requires R40/R41 hand-fitted (section 1) and the Qwiic SHT4x from the ESP32 project kit.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| I2C-01 | Idle levels ~3.3 V on CN1.3 (SDA) / CN1.4 (SCL) — requires R40/R41 4.7 k fitted (BOM gap: hand-kit them first!) | 3.2-3.4 V |  |  |
| I2C-02 | dvt i2c scan with a Qwiic SHT4x attached: 0x44 found at 100 kHz | Found 0x44 |  |  |
| I2C-03 | dvt i2c sht4x: plausible T / RH, CRC OK at 100 kHz | CRC OK |  |  |
| I2C-04 | dvt i2c scan fast + dvt i2c sht4x fast: same results at 400 kHz | OK at 400 kHz |  |  |
| I2C-05 | Rail split proof: remove the J2 jumper — 3V3_NRF dies but the Qwiic device stays powered. V(CN1.2) | 3.20-3.40 V (CN1.2 = 3V3_OTHER) |  |  |

# 14. IMEAS — Current Measurement (J2, 3V3_NRF) — headline feature

The headline feature. J2 jumper out, meter in series (PPK2 ampere-meter mode ideal; note DMM burden voltage on uA ranges). Debug session DETACHED for every floor measurement — DBG holds power domains up (IMEAS-08 demonstrates). Provisional limits become datasheet claims from these first-article numbers.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| IMEAS-01 | Meter burden sanity: meter on mA range in J2, shell idle: V(J2.2) sag vs jumpered | <= 50 mV |  |  |
| IMEAS-02 | Baseline: shell idle (UART active) current on J2 | Record (0.3-8 mA) |  |  |
| IMEAS-03 | dvt cpu: 30 s busy-loop current | Record (3-10 mA) |  |  |
| IMEAS-04 | dvt sleep idle (console suspended, QSPI in DPD, debugger DETACHED): System ON idle floor | <= 25 uA provisional (goal <= 10) |  |  |
| IMEAS-05 | dvt sleep off: System OFF floor (wake = SW4) | <= 10 uA provisional (goal <= 3) |  |  |
| IMEAS-06 | dvt qspi dpd on/off from the idle state: delta visible on the meter (MX25R64 standby vs DPD) | Delta recorded |  |  |
| IMEAS-07 | dvt ble adv 0, default interval: average current (PPK2 / averaging meter) | Record (<= 60 uA) |  |  |
| IMEAS-08 | Debugger influence: repeat the idle measurement with a pyocd/probe-rs session attached — current rises. Rule: always measure detached | Documented |  |  |
| IMEAS-09 | Isolation proof: J2 open — RP2040 CDC alive, LED2 on, Qwiic device powered, while V(3V3_NRF) | <= 0.10 V |  |  |

# 15. RF — Bluetooth LE Radio

Needs the net-core ipc_radio image flashed (Firmware Guide section 4). Same spot, same orientation for every comparative reading.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| RF-01 | Net core flashed (merged_CPUNET.hex); dvt ble adv: advertising starts | Starts |  |  |
| RF-02 | nRF Connect (phone): found as 'nRF5340-DVT'; RSSI at 1 m, phone facing the antenna edge | Record (-70--35 dBm) |  |  |
| RF-03 | RSSI delta vs reference board (nRF5340-DK ideal), same spot, 4 orientations | Within +/- 6 dB |  |  |
| RF-04 | Connect from the phone: GATT device name reads; link holds 2 min | Holds |  |  |
| RF-05 | dvt ble adv 3: RSSI rises vs 0 dBm | +1.5 to +4 dB |  |  |
| RF-06 | Detune sanity: hand over the module drops RSSI, recovers on release (antenna genuinely radiating) | Behaves |  |  |
| RF-07 | Coex soak 5 min: adv + CDC streaming + repeated dvt qspi test: no resets, RSSI stable | Stable |  |  |

# 16. SYS — System / Application Level

Whole-board soak and lifecycle checks. Run these last, battery fitted.

| ID | Check / Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| SYS-01 | dvt demo soak 30 min (SHT4x poll + BLE adv): no reset / watchdog, uptime monotonic | Clean 30 min |  |  |
| SYS-02 | Power-cycle x10 (USB replug): banner every time | 10 / 10 |  |  |
| SYS-03 | Battery-only demo 10 min, with a USB plug/unplug mid-run: no reset | Clean |  |  |
| SYS-04 | Thermal after 30 min active + charging: warmest of U5 / U6 / U7 / U2 | Record (<= 60 degC) |  |  |
| SYS-05 | Post-soak mechanical: SW1..SW4 exercised x20 each, still correct | OK |  |  |
| SYS-06 | Archive: photos, tracker filled, issues logged, disposition + sign-off | Complete |  |  |

# 17. Results Summary & Sign-Off

Fill from the tracker Dashboard when the campaign completes.

| Subsystem | Tests | Pass | Fail | Blocked |
| --- | --- | --- | --- | --- |
| VIS — Incoming Visual & Mechanical Inspection | 11 |  |  |  |
| PRE — Unpowered Electrical Checks (DMM) | 13 |  |  |  |
| PWR — First Power-Up and Power Rails | 14 |  |  |  |
| CHG — Battery Charger (MCP73831-2AC, PROG = 5.1 k) | 6 |  |  |  |
| BAT — Battery Protection and Power Path (DW01A + FS8205A + AO3415/AO3407) | 6 |  |  |  |
| DBG — RP2040 Debug Subsystem (CMSIS-DAP probe + UART bridge) | 10 |  |  |  |
| NRF — nRF5340 Bring-Up (MDBT53-1M) | 9 |  |  |  |
| GPIO — I/O Verification (H1 / H2 headers) | 5 |  |  |  |
| I2C — Qwiic Port (CN1) and I2C1 | 5 |  |  |  |
| IMEAS — Current Measurement (J2, 3V3_NRF) — headline feature | 9 |  |  |  |
| RF — Bluetooth LE Radio | 7 |  |  |  |
| SYS — System / Application Level | 6 |  |  |  |
| Total | 101 |  |  |  |

Issue log: keep the authoritative list in the tracker Issue_Log sheet (Test ID, description, severity, disposition); summarize closed items here at sign-off.

Board disposition:  [ ] PASS   [ ] PASS with deviations   [ ] FAIL

Signature: ____________________    Date: ____________

# Appendix A — Open Items Entering DVT

- R40/R41 (I2C pull-ups) missing from BOM — hand-kit for DVT, fix BOM before pilot (blocking for I2C section).

- FCC/ISED "Contains" marking absent from silk — sale blocker, not DVT blocker; add before pilot fab.

- Raytac antenna keep-out sign-off pending (service@raytac.com); RF-02/03 are the interim empirical gate.

- H1/H2/J2 headers + TC2030-CTX-NL cable are off-BOM kitting items; decide ship-fitted vs ship-loose for the pilot.

- P1.04 stub net + TP silkscreen naming (TP1-3 vs NFC1/2/GND) — schematic/silk housekeeping next revision.

- nRF Connect VS Code Flash/Debug buttons do not support CMSIS-DAP — page-one documentation item for the product (west flash --runner pyocd / probe-rs; Cortex-Debug for debugging).

- Sleep-current provisional limits (IMEAS-04/05/07) to be tightened into datasheet claims from first-article measurements.

# Appendix B — Verified Pin Definitions (firmware)

/* nRF5340 board Rev B — fab 2026-07-04 (netlist-verified) */

/* Console  uart0: TX P0.20 (H1.15), RX P0.22 (H1.14) <- RP2040 bridge  */

/* Qwiic    i2c1: SDA P1.02 (CN1.3/H1.18), SCL P1.03 (CN1.4/H1.17)     */

/* QSPI     mx25r64: CS P0.18, SCK P0.17, IO0-3 P0.13-P0.16 (DK match) */

/* LFXO     X2 NX3215SA CL=6pF, no external caps:                      */

/*          CONFIG_SOC_LFXO_CAP_INT_6PF=y   <- the ONE DK deviation    */

/* Header GPIO (23 free + UART/I2C pins):                              */

/*  H1: P0.29 P0.07 P0.25 P0.26 P0.27 P0.10 P0.09 P0.08 P1.01         */

/*  H2: P0.11 P0.12 P0.05 P0.06 P0.04 P1.00 P1.15 P1.14 P1.12         */

/*      P0.31 P0.28 P1.11 P0.30 P1.10                                  */

/*  AIN0-7 = P0.04 P0.05 P0.06 P0.07 P0.25 P0.26 P0.27 P0.28          */

/* RP2040 (stock debugprobe): GPIO2 SWCLK, GPIO3 SWDIO, GPIO4 UART TX  */

/*  -> nRF RX, GPIO5 UART RX <- nRF TX, GPIO25 LED3. Own SWD not       */

/*  broken out (UF2 update only, SW2+SW3).                             */

# Appendix C — DVT Helper Firmware (pack summary)

One Zephyr application for the nRF5340 app core, shell-driven on the probe CDC (uart0) and RTT simultaneously. Build: west build -b nrf5340dk/nrf5340/cpuapp --sysbuild (NCS >= 2.9). Commands and the DVT IDs they exercise:

- dvt info — FICR S/N, reset reason, LFCLK source (NRF-02/03)

- dvt clk / clk cal — LFCLK status; LFXO ppm vs HFXO (NRF-04/05; catches wrong load caps)

- dvt gpio walk [all] / in / pattern — walking output, pull-up table, adjacent-bridge auto test (GPIO-01/02/03)

- dvt adc — AIN0..7 dump every 500 ms (GPIO-04)

- dvt i2c scan [fast] / sht4x [fast] — Qwiic scan + SHT4x with CRC at 100/400 kHz (I2C-02/03/04)

- dvt qspi id / test / dpd on|off — MX25R64 presence, erase-write-verify, deep-power-down (NRF-06/07, IMEAS-06)

- dvt usb — nRF native USB CDC, SW1 in APP (NRF-08)

- dvt ble adv [0|3] / stop — advertise at 0 or +3 dBm (RF-01..05; needs net-core image)

- dvt cpu — 30 s busy-loop (IMEAS-03)

- dvt sleep idle / off — System ON idle and System OFF with 5 s countdown, console+QSPI suspended (IMEAS-04/05)

- dvt demo — SHT4x + BLE soak loop (SYS-01/03)

RP2040 side runs the stock debugprobe release UF2 — no custom firmware. Record both versions on the tracker Dashboard. Flashing over the onboard CMSIS-DAP probe, the APPROTECT unlock and the net-core procedure are in DVT_Firmware_Guide.docx.